xCL_AT-170-E  7" eCommerce Terminal

#retail #restaurant #hypermarket #grocerystore #foodcourt

SPECIFICATIONS

- PCI PTS 6.x , Android Secure 12 OS

- ARM Cortex-A53 quad-core 64bit

- 2 GB SDRAM +16 GB eMMC

- Can up to 4 GB SDRAM +64 GB eMMC

- 7”inch touch display,  Secure Pin on glass

- 5MP Auto-Focus back camera (up to 8MP)

Security

- PCI PTS 6.x SRED

- Key Management: MK/SK, DUKPT

- TLS 1.2 support

- Secure Code Update Architecture

Certification

- PCI PTS 6.x with SRED

- GBIC, and CC compliant

- EMV Contact L1 & L2

- EMV Contactless L1

- PayPass, PayWave, DPAS, ExpressPay, Quickpass, JSpeedy

- Mastercard TQM

- Sony Felica

Thermal Printer

- 48 mm printing width with tear bar

- 34 mm diameter paper roll

Communication

- WI-FI 802.11 a/b/g/n/ac (2.4GHz & 5GHz)

- Bluetooth 5.0 Dual Mode

- 4G LTE (Support 2 SIM slots)

GPS

- - Tracking GPS (Optional)

Extension Ports

- USB Type C Connector x 1

Battery/ Power

- Typical capacity: 3.7V / 5300mAH

- Rated capacity: 3.7V / 5200mAH

Camera

- 5MP Auto-Focus back camera

- Optional 8MP camera

- Support 1D/2D barcode scanning (Optional)

Dimensions

- 231.1 (L) x 113.75 (W) x 71.76(H) mm

Hand held part thickness 27.5mm

Environmental

- Operating Temperature: 0C to 50C

- Storage Temperature: - 20C to 60C

- Operating Humidity: 5% to 85% (non-condensing)

Accessory

- DC switching power adapter (5V/2A)

- USB cable (type C to type A)

- Rechargeable battery

- Paper roll (34mm diameter)

- Quick user guide/safety instruction

- Privacy screen protector (Optional)

- Water resistant printer cover (Optional)

- Visual impaired accessory (Optional)

- Charging Base Device (Optional)

  - XC70-E (Ethernet + USB-A x 2+RS232 x 2)

  - XC70-C (Charge Only)

- Communication Box (Optional)

- USB2.0 type A x2

- USB2.0 type C x1 (device port)

- RJ-45 Ethernet (10/100) x1

- RJ-12 COM x1

- MicroSD card slot x1

- 12V Power DC Jack x1

- USB display dongle (Optional)

- Drive 2nd touch display to support

● ECR POS/KISOK/Vending Machine

Customer Facing Touch Display

CPU

- Qualcomm ARM Cortex-A53 quad-core 64bit (QCM2290) up to 2.0GHz

- Secure: MAXIM ARM Cortex-M3 SoC

Operating System

- Android 12

Memory

- 2 GB SDRAM +16 GB eMMC

Optional

- 3 GB SDRAM +32 GB eMMC

- 4 GB SDRAM +64 GB eMMC

Display

- 7” 1280x800 color TFT

- Capacitive touch screen display

AUDIO

- Audio Jack and Microphone

PINPAD

- Touch PINPAD

Magnetic Stripe Reader

- ISO 7811, 3-tracks

Smart Card Reader

- ISO7816-1/2/3 asynchronous T=0, T=1

- SLE4442 Memory Card

- Support 2 SAM slots

Contactless Reader

- ISO14443 type A/B

- ISO18092 (NFC)

- MiFare

- Felica